Grant Roberts

MP/NTP 651 Fall 2020

fMRI Analysis - Due 10/27/2020

- List 3 common preprocessing steps in fMRI and briefly describe why each of these steps is performed.

1. Motion correction – Slight changes in head motion can artificially increase signal near the edges of the brain, making it appear as if activation occurred due to the sharp change in signal. Registration of each sub-brick can be performed to adjust correct for this.

2. Skip the first few images – When performing gradient echo sequences, spins take some finite amount of time to go from fully-magnetized (|| to B0) to “steady-state”. The signal in the first few images (for every voxel) will thus be increased, which can adversely affect the statistical comparisons. To correct for this, we simply throw the first few images out (given that dummy scans are not automatically performed in the sequence).

3. Field map correction – Because the magnetic field lines change when a heterogenous material is placed in the scanner, we can have anywhere from minor to severe distortion/signal loss within our image. We can adjust for “warping” by obtaining a field map prior to imaging, which will allow us to determine the degree and direction of distortion and retrospectively correct for this within the image.

- Let’s say you want to study the brain activation from a task with 3 conditions (happy faces, neutral faces, fearful faces) in addition to a baseline (fixation). You decide to present the data in the following blocked design:

Each block consists of 10 faces, one face presented every 2 seconds (and lasting 2 seconds in duration).

- What ideal regressors (i.e. regressors of interest) would you include to model the activation to these conditions?

You would have 3 regressors of interest: β1 (happy), β2 (fear), and β3 (neutral) designed to model the activation amplitudes of each task. These are calculated by temporally convolving the HRF with the block design for each specific task and applying regression to calculate each beta coefficient.

- How would you test the difference between the response to happy and fearful faces?

β1 - β2 ≠ 0  or  β2 – β1 ≠ 0

- Say you are running an fMRI data analysis, and the analysis program tells you that you have a problem with “multi-collinearity”. What is multi-collinearity? How would you fix this problem?

Multi-collinearity is essentially a problem related to rank-deficiency, meaning that the task designs are not completely independent from each other. In other words, some linear combination of task designs is equal to another task design or a constant. This makes it impossible to disentangle that design from the combination of the others.  The solution would be to choose one condition as the control state and compare the other regressors to this control regressor.

- What is one advantage of a blocked design for fMRI?

Blocked designs have the advantage of simplicity/robustness, large BOLD signal changes relative to baseline, and increased statistical power (Tie Y, et al. Neuroimage 2009).

- What is one advantage of an event-related design for fMRI?

Event-related designs may be more appropriate for specific tasks (such as swallowing), can be unpredictable in nature (less neural adaptation), can allow for sorting of the events based on a participants response, maintains subject’s attention, and is less sensitive to task-induced motion such as speaking.