Title of Project: Simultaneous Assessment of Tissue Microstructure and Cerebrovascular Disease in Alzheimer’s Disease using 4D Flow MRI and Neurite Orientation Dispersity and Density Imaging

Authors: Grant S. Roberts, MS

Abstract: The primary goal if this project is to investigate the effects of cerebrovascular disease (as measured by 4D flow MRI) on microstructural changes (as measured by NODDI) in a cohort of AD, MCI, and control subjects. The first aim of this study is to: Develop a fully-automated NODDI processing pipeline in FSL and Python from multi-shell diffusion tensor imaging (DTI) data in order to produce NDI, ODI, and Viso maps. The second aim of this study will be to perform NODDI processing in 28 AD, 34 MCI, and 42 CN individuals. This will be matched to the cerebrovascular data that has already been calculated for these subjects. Data will be analyzed using ANCOVA for normally distributed data and Kruskal-Wallis for non-normal data. Group differences will be explored using non-parametric permutation tests with threshold-free cluster enhancement. Spearman correlation analyses will be performed on each NODDI measure with cerebrovascular disease metrics for each group.

Introduction:

While Alzheimer’s disease (AD) has been widely associated with parenchymal and microstructural alterations2, it is also strongly linked to cerebrovascular disease (CVD) including cerebral hypoperfusion3, small vessel disease4, and blood brain barrier dysfunction5. Still, the potential synergistic or even causative relationship between cerebrovascular health on observed structural brain changes is unclear6. Studying this relationship is partly limited by the lack of sufficiently sensitive tools to evaluate the many facets of vascular pathology.

Over the last decade, the UW MRI research group in collaboration with the Wisconsin Alzheimer’s Disease Research Center has developed novel methodologies for the quantitative assessment of both cerebrovascular health and parenchymal changes on the microstructural level. Our cardiovascular research group has pioneered a method to comprehensively evaluate macroscopic cerebrovascular hemodynamics in AD using time-resolved, volumetric, velocimetry measurements (4D Flow MRI7,8) with a high-resolution, radially-undersampled acquisition and have identified alterations in mean blood flow and pulsatility9-13. Recently, a novel technique termed ‘neurite orientation and density imaging (NODDI)’ has been introduced as a method to probe microstructural complexity on a voxel-wise basis14. NODDI works by fitting data from a multi-shell diffusion tensor imaging (DTI) into an a priori three-compartment tissue model, allowing for robust measurements of dispersion indices (ODI), neuronal density indices (NDI), and volume fractions of isotropic water molecules (Viso)15. To date, only a handful of studies have evaluated tissue microstructural changes in AD cohorts using NODDI16,17; each independently concluding that decreases in ODI and NDI occur in various cortical and subcortical structures in individuals with AD18-22. However, there have been no studies in which simultaneously assess CVD and microstructural tissue alterations in AD subjects. It is hypothesized that measures of CVD (as measured by intracranial pulsatility) will be positively correlated to white matter degradation (as measured by NDI). We further hypothesize that a stronger relationship will be observed as AD severity increases.

Materials and Methods:

Subjects

A total of 28 AD and 42 age-matched controls have been identified from the ADRC and ADRC/WRAP protocols. These specific cases have been analyzed by Dr. Leonardo Rivera-Rivera for pulsatility index (PI), pulse wave velocity (PWV), and cerebral blood flow (CBF) measurements in the internal carotid artery (ICA)13. The use of these measures has been discussed and approved by Dr. Rivera-Rivera. These data have already been pre-tabulated for each patient. Data will be transferred from the ADRC servers to Medical Physics servers (data_mrcv2 drive) for FMRIB Software Library (FSL) processing. Once data has been transferred, NODDI datasets will be inspected for image and gating quality and will be organized for the pre-processing steps listed below. Note that only NODDI datasets will be transferred for this analysis, as the 4D flow cases have already been processed.

MRI Data Acquisition and Preprocessing

MRI data were acquired using a clinical 3.0T scanner (Discovery MR750, GE Healthcare, Waukesha, WI) with a 32-channel head coil (Nova Medical, Wilmington, MA). T1-weighted structural images were acquired using a 3D inversion recovery prepared fast spoiled gradient-recalled echo (BRAVO) sequence (TI=450 ms; TR/TE=8.2/3.2 ms; flip angle=12°; 1×1×1 mm3 voxel resolution; 256×256×[156-192] acquisition matrix). A multi-shell spin-echo echo-planar imaging pulse sequence was used to acquire 69 total diffusion-weighted images spread across 4 q-space shells (6×b=0 s/mm2, 9×b=500 s/mm2, 18×b=800 s/mm2, and 36×b=2000 s/mm2; TR/TE=8575/76.8 ms; 2×2×2 mm3 isotropic resolution; 128×128×74 acquisition matrix).

A preprocessing pipeline was implemented in FSL. Raw diffusion-weighted images were denoised using dwdenoise and corrected for Gibb’s ringing artifact using mrdegibbs. Skull-stripping was accomplished using BET, (5) motion correction using FLIRT and eddy correction using eddy, (6) perform bias field corrected with N4BiasFieldCorrection, (7) and smooth the DWI data fslmaths. For motion correction, affine registration will be used to register each DWI image to the b0 image. This is a similar pipeline to that described by Maximov, et al.23

Diffusion tensor images (DTIs) will then be estimated from the pre-processed data. Finally, the Dmipy python package1 will be used to model NODDI parameter maps from the DTIs. DTI volumes will be spatially normalized to a population-specific template using DTI-TK (http://dti-tk.sourceforge.net) which bootstraps the input tensor volumes and aligns them to the template in an iterative fashion24 with a tensor-based registration algorithm25. This framework has been shown to improve white-matter alignment compared to conventional fractional anisotropy registration methods26. Tract-based Spatial Statistics from FSL will be used to detect whole-brain white matter differences between AD, MCI, and CN groups with PWV, pulsatility index (PI), and mean cerebral blood flow as covariates. Additional covariates include: age, sex, systolic blood pressure, diastolic blood pressure, and heart rate.

Data will be analyzed using ANCOVA for normally distributed data and Kruskal-Wallis for non-normal data. Group differences will be explored using non-parametric permutation tests (N=5,000) with threshold-free cluster enhancement. Spearman correlation analyses will be performed on each NODDI measure with cerebrovascular disease metrics for each group.

Results:

Discussion:

Conclusion:

Bibliography and References Cited:

1	Fick, R. H. J., Wassermann, D. & Deriche, R. The Dmipy Toolbox: Diffusion MRI Multi-Compartment Modeling and Microstructure Recovery Made Easy. Front Neuroinform 13, 64, doi:10.3389/fninf.2019.00064 (2019).

2	Vogt, N. M. et al. Cortical Microstructural Alterations in Mild Cognitive Impairment and Alzheimer's Disease Dementia. Cereb Cortex, doi:10.1093/cercor/bhz286 (2019).

3	Roher, A. E. et al. Cerebral blood flow in Alzheimer's disease. Vasc Health Risk Manag 8, 599-611, doi:10.2147/VHRM.S34874 (2012).

4	Kester, M. I. et al. Associations Between Cerebral Small-Vessel Disease and Alzheimer Disease Pathology as Measured by Cerebrospinal Fluid Biomarkers. JAMA Neurology 71, 855-862, doi:10.1001/jamaneurol.2014.754 (2014).

5	Sweeney, M. D., Sagare, A. P. & Zlokovic, B. V. Blood-brain barrier breakdown in Alzheimer disease and other neurodegenerative disorders. Nat Rev Neurol 14, 133-150, doi:10.1038/nrneurol.2017.188 (2018).

6	Santos, C. Y. et al. Pathophysiologic relationship between Alzheimer's disease, cerebrovascular disease, and cardiovascular risk: A review and synthesis. Alzheimers Dement (Amst) 7, 69-87, doi:10.1016/j.dadm.2017.01.005 (2017).

7	Johnson, K. M. et al. Improved 3D phase contrast MRI with off-resonance corrected dual echo VIPR. Magn Reson Med 60, 1329-1336, doi:10.1002/mrm.21763 (2008).

8	Gu, T. et al. PC VIPR: a high-speed 3D phase-contrast method for flow quantification and high-resolution angiography. AJNR Am J Neuroradiol 26, 743-749 (2005).

9	Clark, L. R. et al. Macrovascular and microvascular cerebral blood flow in adults at risk for Alzheimer's disease. Alzheimers Dement (Amst) 7, 48-55, doi:10.1016/j.dadm.2017.01.002 (2017).

10	Xu, G. et al. Reliability and precision of pseudo-continuous arterial spin labeling perfusion MRI on 3.0 T and comparison with 15O-water PET in elderly subjects at risk for Alzheimer's disease. NMR Biomed 23, 286-293, doi:10.1002/nbm.1462 (2010).

11	Rivera, L. et al. Changes in intracranial venous blood flow and pulsatility in Alzheimer’s disease: A 4D flow MRI study. Journal of Cerebral Blood Flow & Metabolism 37, 0271678X16661340-10271678X16661340, doi:10.1177/0271678X16661340 (2016).

12	Rivera-Rivera, L. A. et al. Assessment of vascular stiffness in the internal carotid artery proximal to the carotid canal in Alzheimer’s disease using pulse wave velocity from low rank reconstructed 4D flow MRI. Journal of Cerebral Blood Flow & Metabolism, 0271678X20910302, doi:10.1177/0271678X20910302 (2020).

13	Rivera-Rivera, L. A., Johnson, S. C., Illingworth, C. & Johnson, K. M. in Proc. Intl. Soc. Mag. Reson. Med. 27.

14	Zhang, H., Schneider, T., Wheeler-Kingshott, C. A. & Alexander, D. C. NODDI: practical in vivo neurite orientation dispersion and density imaging of the human brain. Neuroimage 61, 1000-1016, doi:10.1016/j.neuroimage.2012.03.072 (2012).

15	Kamiya, K., Hori, M. & Aoki, S. NODDI in clinical research. Journal of Neuroscience Methods 346, 108908, doi:https://doi.org/10.1016/j.jneumeth.2020.108908 (2020).

16	Sone, D.

17	Kamagata, K. et al. Advanced diffusion magnetic resonance imaging in patients with Alzheimer&#39;s and Parkinson&#39;s diseases. Neural Regeneration Research 15, 1590-1600, doi:10.4103/1673-5374.276326 (2020).

18	Fu, X. et al. Microstructural White Matter Alterations in Mild Cognitive Impairment and Alzheimer's Disease : Study Based on Neurite Orientation Dispersion and Density Imaging (NODDI). Clin Neuroradiol 30, 569-579, doi:10.1007/s00062-019-00805-0 (2020).

19	Parker, T. D. et al. Cortical microstructure in young onset Alzheimer's disease using neurite orientation dispersion and density imaging. Hum Brain Mapp 39, 3005-3017, doi:10.1002/hbm.24056 (2018).

20	Vogt, N. M. et al. Cortical Microstructural Alterations in Mild Cognitive Impairment and Alzheimer's Disease Dementia. Cereb Cortex 30, 2948-2960, doi:10.1093/cercor/bhz286 (2020).

21	Slattery, C. F. et al. ApoE influences regional white-matter axonal density loss in Alzheimer's disease. Neurobiol Aging 57, 8-17, doi:10.1016/j.neurobiolaging.2017.04.021 (2017).

22	Wen, Q. et al. White matter alterations in early-stage Alzheimer's disease: A tract-specific study. Alzheimers Dement (Amst) 11, 576-587, doi:10.1016/j.dadm.2019.06.003 (2019).

23	Maximov, I. I., Alnæs, D. & Westlye, L. T. Towards an optimised processing pipeline for diffusion magnetic resonance imaging data: Effects of artefact corrections on diffusion metrics and their age associations in UK Biobank. Human Brain Mapping 40, 4146-4162, doi:10.1002/hbm.24691 (2019).

24	Zhang, H. et al. High-dimensional spatial normalization of diffusion tensor images improves the detection of white matter differences: an example study using amyotrophic lateral sclerosis. IEEE Trans Med Imaging 26, 1585-1597, doi:10.1109/tmi.2007.906784 (2007).

25	Zhang, H., Yushkevich, P. A., Alexander, D. C. & Gee, J. C. Deformable registration of diffusion tensor MR images with explicit orientation optimization. Med Image Anal 10, 764-785, doi:10.1016/j.media.2006.06.004 (2006).

26	Wang, Y. et al. DTI registration in atlas based fiber analysis of infantile Krabbe disease. NeuroImage 55, 1577-1586, doi:10.1016/j.neuroimage.2011.01.038 (2011).