Grant Roberts

MP/NTP 651

09/08/2020

HOMEWORK 1

- Why do we need a large magnet in MRI? (In other words, what does the large magnetic field do to the body, which we exploit to form an MR image)?

A large magnetic field allows for a larger distribution of spins that exist in the higher energy states (according to the Boltzmann distribution). This increased distribution in higher energy spin states allow for an increased net magnetization vector (from a classical perspective) aligned with the main magnetic field. This then allows for more magnetization to be tipped into the transverse plane, which then leads to a higher voltage induced in the RF coils and a greater signal from the sample.

- Why do we need to send radio-frequency waves into the body (or object being imaged) to create an MR image?

RF waves tip the net magnetization vector into the transverse plane. Assuming the magnetization vector has some finite length, nuclear precession will cause spins to precess about the Z-axis at the Larmor frequency. Again, this causes a voltage to be induced in the RF coils (Faraday’s Law).

- Why do we need a magnetic field gradient in order to create an MR image?

Magnetic field gradients allow us to spatially resolve spins at different locations. This is because, when a magnetic field gradient is applied, spins at different locations along the gradient will experience different precessional frequencies. After spin excitation, these differences in frequency will present themselves in our measured signal. We can resolve these frequencies and amplitudes by applying the Fourier transform to the FID/echo signal, and because the frequencies of the signal are related to space via the imaging gradients, we can form an image.

- If I decrease the repetition time (TR) and keep all other parameters the same, does the MRI signal (a measure of the net magnetization) increase or decrease? Why?

Assuming a 90-180 degree spin echo sequence, the MR signal will decrease on subsequent excitations because we are decreasing the amount of regrowth of the longitudinal aspect of the magnetization vector caused by T1 relaxation. Because the we have decreased magnetization in the z-component, there will be less transverse magnetization after the following 90-degree excitation which will lead to less signal induced in the RF coils.

- If I increase the echo time (TE), does the signal increase or decrease? Why?

Assuming a 90-180 degree spin echo sequence, MR signal will decrease due to a decay in the transverse magnetization caused T2 relaxation. As we wait longer to readout our signal, this leads to increased time for our spins to dephase, hence, decreasing our transverse magnetization leading to less signal induced in the RF coils.