Grant Roberts

MP/NTP 651

09/16/2020

Homework 2 – MRI Pulse Sequences

- Say that the signal-to-noise ratio (SNR) of an image where all of k-space is acquired is 100. If we acquire only half of the k-space and fill in the rest using complex conjugate symmetry, what is the SNR of this partial k-space acquisition?

The SNR would be reduced by a factor of 2, and would thus be =1002=70.7

- How can “parallel imaging” acquire the same resolution image as a conventional (non-parallel) acquisition in a shorter amount of time?

When coherently undersampling phase encode lines in k-space, we can reduce scan time by a factor proportional to the degree of undersampling. However, when doing so, we generate what are called ‘wrap-around’ artifacts in which the image folds into itself. When this occurs, it is impossible to visually distinguish between the true signal and wrapped signal. However, one can employ parallel imaging by utilizing known spatial sensitivity information from multiple receiver coils. This information can be used to determine the contribution of signal from each receiver array. Linear algebra can then be used to determine the location of the signal from the ‘wrapped’ part of the image and from the ‘true’ part of the image. This allows to digitally expand our field of view and fill in the missing information from undersampling, which is how we can acquire the same resolution in a shorter amount of time. A simplified way to think of this is that one can make a crude prediction of where the signal is coming from by looking at the relative intensities of images from each separate coil, because signal falls off over distance. In essence, by gathering additional information from more coils, we accelerate imaging times, however this does with a reduction in signal to noise ratio.

- Why do most 2-dimensional MRI imaging sequences use interleaved rather than sequential slice acquisition?

When we excite a 2D slice, our RF pulses are not perfect rectangular slabs (which would require an infinitely long sinc pulse). Since we can’t image forever, we instead excite a slice that is not exactly rectangular. This means that some of the excitation will bleed over into our next excitation location (called cross-talk). This will push some of the longitudinal magnetization from neighboring slices into the transverse plane. If we were to sequentially acquire slices, we would result in reduced transverse magnetization. By interleaving slices, we can move to a more distant slice, which has been minimally affected by cross-talk. This allows for a full longitudinal magnetization vector to be pushed into the transverse plane, leading to greater signal. After completing the first interleave, we can then come back to the neighboring slices that were skipped which should have a nearly fully-recovered longitudinal magnetization (depending on the T1 of the tissue) to excite.

- Echo planar images (EPI) are a common, if not the most common, type of image acquired in functional MRI. a) What is one advantage of echo-planar imaging (EPI) compared to “conventional” (non-EPI) gradient-echo imaging? b) What is one disadvantage?

a.)  We can image very fast by traversing k-space in one shot, allowing one to acquire more images per unit time. This is useful for fMRI because we need to detect brain activity in potentially very short time intervals.  Conventional GRE would require RF pulses every readout, which would slow the acquisition down.

b.)  Imaging fast with very long readouts can lead to severe image distortion and signal loss caused by field inhomogeneities.

- What causes the signal dropout near the front of the brain when acquiring a 2-dimensional axial (gradient-echo) echo-planar image?

There is an abrupt difference in susceptibility between the air-filled sinuses and the surrounding bone, leading to local field inhomogeneities. This will increase the T2* dephasing in spins within this region. Because we are using a gradient echo EPI, T2* decay will result in signal loss for the entire readout.