Grant Roberts

MP/NTP 651 Fall 2020

Diffusion Tensor Imaging Homework - Due 10/7/2020

- Does the apparent diffusion coefficient of water increase or decrease as the cellularity (cellular density) of tissue increases?  Why?

The apparent diffusion coefficient (ADC) decreases with increasing cellularity. This is because the path of diffusion of water molecules becomes physically more restricted as the number of surrounding cells increase.

- The diffusion gradient pulse parameters for a diffusion-weighted imaging sequence are

G=30 mT/m

d=10 ms

D=30 ms

How does the b-value change if

G=50 mT/m

d= 8 ms

D=25 ms

The equation to calculate b-values for rectangular gradient pulses is given as: b=γ2G2δ2Δ-δ3.

b0=γ230 mTm210 ms230 ms-103 ms=γ2*2.400*106ms3mT2m2

b1=γ25028225-83= γ2*3.573*106ms3mT2m2

The b-value will increase by ~1.5 times for the second acquisition.

- Is the diffusion-weighted signal higher when the diffusion gradients are applied in the direction parallel to the white matter tract or in the perpendicular direction?  Why?

Due to the fact that axons are long, thin, cylindrical tissues, water that is diffusing perpendicular to the length of the tract (radial direction) will be restricted. This decrease in diffusion will cause signal to increase because there is less dephasing of spins caused by the bipolar gradients. The lower the amount of spin dephasing, the greater the transverse magnetization when reading out the MR signal.

- How much will the echo planar image distortion change if the echo spacing increases by 30% and the field of view is reduced by 50%?  What happens to the distortion if the phase encoding direction is rotated by 90 degrees?

The formula for EPI distortion is given by: Δy=ΔB0*esp*FOV/R.

If we arbitrarily assume that all values were initially 1, then: Δy=1⋅1.3⋅0.5⋅1=0.65. This implies that the distortion decreases by 35%. If we rotate that phase encoding direction, then the same degree of distortion will occur, but in the x-direction. In other words, the distortion will occur but only along the direction in which the phase encodings were acquired.

- What are the eigenvalues and eigenvectors of the diffusion tensor?  What do they represent and how are they modulated?

The eigenvectors of the 3x3 diffusion matrix are orthogonal vectors that point in the principal diffusion directions, which define the ‘local fiber coordinates system’ in anisotropic tissues. The positive eigenvalues (λ1,λ2,λ3) give degree of diffusivity in the direction of each eigenvector. These are modulated directly by the morphology and diffusion properties of tissue microstructures, for instance, white matter is composed of axons which are long, narrow, cylindrical cells which would result in an eigenvector along an axis parallel to the length of an axon and a high eigenvalue (diffusivity) along this axis.

- What are the FA and Trace of the diffusion tensor if

l1 = 1.1x10-3 mm2/s

l2 = 0.8x10-3 mm2/s

l3 = 0.7x10-3 mm2/s

Trace=λ1+λ2+λ3=2.6*10-3 mm2/s

FA=12λ1-λ22+λ2-λ32+λ3-λ12λ12+λ22+λ32=0.236

- What is the diffusion-weighted signal if

So = 100

b = 1000 s/mm2

D = 0.8x10-3 mm2/s

What do each of these terms represent?

The So term represents the voxel-wise b-zero signal (T2-weighted EPI image).

The b term represents the calculated b-value from the specific diffusion gradient waveforms used for imaging.

The D term is the diffusion coefficient and is a measure of the strength of diffusion, assuming water diffuses freely and in a Gaussian distribution.

The diffusion weighted signal would be: SDWI=S0e-bD=100⋅e-1000smm2*0.8*10-3mm2s=44.9